ALTER

Oscilloscope

Time-Domain Waveform Scope

Oscilloscope displays sound as waveform motion through time: shape, symmetry, clipping, punch and stereo movement.

Oscilloscope is ALTER's time-domain analyzer. Spectrum tells you where the frequencies live. Audio Meters tell you how loud, safe or compressed the signal is. Oscilloscope shows the actual waveform: the shape of the sound moving through time.

This is the module for the moment when you stop guessing and ask: what the hell is the waveform actually doing? Is the kick clean? Is the bass symmetrical? Did distortion add character, or did it just chew the waveform into digital gravel?

Meters read energy. Oscilloscope reads shape.

# What it actually measures

A waveform is amplitude over time. It is not a spectrum, and it is not a loudness target.

In simple language: Oscilloscope draws the audio waveform. Vertical movement is amplitude; horizontal movement is time. A sine wave, a clipped kick, a noisy bass, a stereo-wide pad and a smashed master all leave different visual fingerprints.

Important: Short-Term Scope is shape-first, not loudness-first. Depending on the visual mode and zoom, the waveform may be normalized or scaled for readability. Do not use its height as a calibrated dBFS value. Use Audio Meters for level truth; use Oscilloscope for shape truth.

# Visual overview: the two scope modes

Short-Term Scope is the microscope. Long Waveform is the timeline. Level History moved to Audio Meters.

| Mode | What it shows | Best use |
| --- | --- | --- |
| Short-Term Scope | A small waveform window, roughly milliseconds long. It is best for seeing the actual cycle or transient shape. | Synth waves, bass cycles, kicks, snares, distortion, polarity checks and stereo timing. |
| Long Waveform | A longer waveform timeline. Zoomed out, it behaves like a DAW-style min/max waveform overview. | Groove shape, phrase movement, transient groups, impacts, arrangement density and waveform blocks. |

# Short-Term Scope - the waveform microscope

Short-Term Scope in Stereo display: fast waveform inspection over a small time window.

Short-Term Scope is the closest mode to a classic oscilloscope: a small slice of the waveform, stabilized enough to inspect shape instead of watching the whole thing slide around like a drunk snake. It is excellent for bass cycles, synth design, kick transient shape, distortion edges and stereo timing.

Use it when you care about waveform shape more than level. A sine should look smooth. A clipped transient may show flat tops. A distorted bass may look folded, chewed or squared. If the wave suddenly looks like a staircase from hell, you probably did something aggressive - maybe genius, maybe terrible.

Bass and synth design: see whether the shape is sine-like, square-like, saw-like, folded or clipped.

Distortion decisions: see whether saturation adds shape smoothly or just chops the waveform into a brick.

Transient inspection: kicks, snares and impacts reveal attack shape better than a normal level meter.

Polarity and stereo sanity: left and right moving opposite ways can warn you before mono playback ruins your day.

Practical zoom tip: about 5-30 ms is often the sweet zone for musical waveform inspection. Ultra-short windows below about 1 ms can look cool, but do not over-interpret them unless the signal is very high-frequency.

# Long Waveform - the timeline view

Long Waveform view: raw waveform behaviour over a longer time window.

Long Waveform is the stretched-out version of the waveform world. Instead of staring at one tiny cycle, you watch the audio breathe across seconds. It is excellent for rhythmic density, transient groups, impacts, gaps, waveform blocks and stereo movement over time.

When zoomed out, a real sample-by-sample line would turn into unreadable spaghetti. So the display behaves like a DAW waveform overview: many samples are represented as a visible min/max shape. That is the correct way to show long waveform detail without making the screen vibrate like it drank six coffees.

Use around 0.5-2 s to inspect groove shape, transient clusters and short musical gestures.

Use around 5-10 s to watch phrases, build-ups, breakdowns and rhythmic density.

Use around 10-30 s to inspect arrangement-level waveform blocks and whether the track becomes one endless wall of pressure.

# How to actually use Oscilloscope

| Situation | What to look for | Meaning |
| --- | --- | --- |
| Clean bass or synth tone | Stable repeating shape, smooth curves or intentional sharp edges. | You are reading the oscillator/wavetable/distortion character directly. This is sound design x-ray vision. |
| Kick or snare transient | Fast attack, flat top, asymmetry, ringing after the hit. | Shows whether the transient punches, clips, rings or gets flattened by processing. |
| Stereo waveform | Left and right shapes moving together or fighting each other. | Useful for polarity, stereo timing and mono-compatibility sanity checks. |
| Distortion/saturation | Edges becoming sharper, flatter or folded. | Shows whether processing adds character or just turns the waveform into toasted geometry. |
| Long musical phrase | Blocks, gaps, repeated transient groups and changing density. | Great for seeing groove structure and arrangement movement before opening a DAW timeline. |

# Controls

Oscilloscope controls focus on waveform display, not loudness targets.

Mode chooses the question. Short-Term Scope asks what the waveform looks like right now. Long Waveform asks how the waveform behaves across a longer window. If you need peak/RMS history, switch to Audio Meters and use Level History there.

Display controls how channels are drawn. Mono is clean and simple. Stereo is better when you need to compare left and right movement. Zoom or Window Length changes the time range. Smooth calms the motion. Fill changes visual body. Rotate and Color are visual/performance controls.

# Naming and scientific accuracy review

| Name / mode | Status | Why |
| --- | --- | --- |
| Oscilloscope | Analyzer | An oscilloscope displays a signal over time. That is exactly what this module does. |
| Oscillator | Generator | An oscillator generates a periodic signal. This module does not generate audio; it visualizes it. |

Scientific caution: do not see Oscilloscope as a calibrated loudness meter. It is a waveform scope. Its job is shape, timing, symmetry, clipping and movement. For RMS, LUFS, True Peak and Level History, use Audio Meters.

# Common mistakes

Using Short-Term Scope as a loudness meter. Do not. That job belongs to Audio Meters.

Thinking a pretty waveform means a good mix. A beautiful waveform can still sound like shit.

Calling every flat waveform clipping. Some synths and distortion designs are meant to be flat or squared. Context matters.

Ignoring stereo movement. Wide is not automatically better; sometimes it is just phase trouble with a nice outfit.

Using one zoom level for everything. Short windows answer cycle questions; long windows answer structure questions.

# Final interpretation

Oscilloscope is the module that makes audio physical. You stop seeing sound as an invisible mix decision and start seeing it as motion: cycles, pressure, symmetry, edges, impact and waveform structure.

For engineers, it is a diagnostic waveform scope. For DJs and VJs, it is a living signal trace. For sound designers, it is a sculpting window. For beginners, it is one of the fastest ways to understand that audio is not magic dust; it is movement - Vibration.

# Sources and basis

Tektronix — Oscilloscope Basics:

Used as the main reference for standard oscilloscope interpretation. Tektronix defines an oscilloscope as an instrument that draws a graph of an electrical signal and explains that the graph shows how the signal changes over time, with time on the horizontal axis and voltage/amplitude on the vertical axis. This supports the Oscilloscope module as a time-domain waveform analyzer, not a loudness meter. :contentReference[oaicite:0]{index=0}

Tektronix — Waveform and Signal Analysis:

Used for waveform-shape interpretation: oscilloscopes reveal time values, signal frequency, distortion, noise, sudden changes, pulses, sine waves, square waves and complex waveforms. This supports the documentation sections about transient behaviour, waveform shape, stereo motion and signal movement. :contentReference[oaicite:1]{index=1}

ITU-R BS.1770-5:

Used as the reference for why loudness and true-peak analysis belong to metering tools, not to the Oscilloscope module. ITU-R BS.1770-5 specifically defines algorithms for measuring audio programme loudness and true-peak audio level. :contentReference[oaicite:2]{index=2}

EBU Tech 3341 — EBU Mode Loudness Metering:

Used as the broadcast/professional metering reference. It describes loudness metering, Momentary / Short-term / Integrated time scales, Loudness Range and Maximum True Peak Level. This supports the separation between ALTER Oscilloscope and ALTER Audio Meters. :contentReference[oaicite:3]{index=3}

ALTER internal Oscilloscope behaviour:

Used for product-specific behaviour: display modes, smoothing, stereo display options, zoom/window behaviour, colour controls and visual style. These are documented as ALTER product behaviour, not as external compliance standards.